EPS 708 / EPS 815 with KMA or KMM

Cleaning of volume meter!

Diese Information soll zusätzlich bereitgestellt werden in der Sprache  Englisch

Dirt or metallic particles which find their way into the measuring cell can lead to failure.

Particles in the gear pump: No measurements, as gear pump is blocked. Supply hose to the volume meter is completely filled with oil.

Particles in the control piston: Bar charts and measurements fluctuate considerably. Test oil level in the supply hose to the volume meter rises and falls.

It is not necessary to remove the measuring cell in order to clean the control piston or the gear pump.

Only the volume meters 1 687 234 009 (KMM and KMA) and 1 687 234 011 (KMA and EPS 708) with 2 securing clips may be disassembled and cleaned. The old version of the volume meter 1 687 234 006 with 2 notches and securing clip may not be disassembled as that would render the calibration invalid.

Disassembly/assembly

- Remove the cover from the hydraulics compartment.

- Remove the plastic cover (6) as well as the LED fixture (11).

3.	Remove volume meter cap (7) (4 screws) and lead LED with fixture (11) and connection cable through the volume meter cap.

4.	Carefully remove the cylinder (2) and the control piston (3). Remove control piston.

!	If there are grooves on the control piston, the volume meter must be replaced.

5.	Rinse the cylinder (2) and the control piston (3) in clean test oil and check the surfaces for scoring.

6.	Insert the control piston (3) into the cylinder (2). Mind the mounting position; the control piston cap (4) must be positioned on the right.

7.	Check that the control piston moves easily by gently moving the cylinder (2).

There are two types of internal gears (10), one with a pressed in driver pin or one with a loose driver pin (12).

Please take care not to lose the loosely inserted driver pin (12) when disassembling and assembling the internal gear (10).

8.	Remove external gear (9), internal gear (10) and sickle (8) with tweezers, clean in clean test oil, and reinsert. Mind the installation position!

After reinsertion internal gear, sickle and external gear must be aligned evenly with the volume meter housing.

9.	Insert the cylinder (2) and control piston (3). Mind the installation position!

10.	Replace O-ring (5) (spare parts set: 1 687 010 136) and check if correctly positioned in the groove.

11.	Push LED with socket (11) and connection cable through opening of the volume meter cap.

12.	Fit LED with socket in plastic cap (6). Mind the installation position!

13.	Replace O-ring (1) (spare parts set: 1 687 010 136) and check if correctly positioned in the groove.

14.	Attach plastic cap (6); ensure that LED with socket is centered in window of flow sensor cap (7).

15.	Carefully fit volume meter cap (7) and fixate with the screws (4 screws). Tightening torque: 11 Nm

16.	Vent volume meter (see next section).

Venting the volume meter

KMA

- Clamp and connect pump (in-line injection pump, distributor injection pump, or oder common rail pump).

Connect KMA and pump hydraulically in such a way that the entire flow rate is channeled directly to the volume meter to be bled.

- Switch on EPS 815 and select pump in EPS 945 software.

With common rails it must be ensured that the flow rate of 30 l/h per measuring channel is not exceeded. If necessary, divide flow rate or select a test step with low flow rate (e.g. Start). The flow rate can be increased by increasing the speed.

- Select test step with high flow rate and start testing.

- Wrap cloth around hose leading to volume meter B1 (Fig. 1, Pos. 1) or volume meter B2 (Fig. 2, Pos. 1) respectively.

- Pinch off this hose 5x consecutively for one second with crimping pliers and re-open. The created interruption and subsequent increase of the flow cause significant movement of the control piston in the flow meter and bleed the measuring system.

- The volume meter is now bled.

Fig. 1: Volume meter B1						Fig. 2: Volume meter B2

EPS 708

- Clamp and connect common rail pump, e.g. 0 445 020 105.

- Switch on EPS 708 and select pump in EPS 945 software.

- Select test step "Start" and start testing.

- Increase speed until flow rate reaches 30 l/h.

- Visually inspect the hose (Fig. 3, Pos. 2) of the flow meter to see whether the respective flow rate passes through the flow meter. If the flow rate is not running through the flow meter, decrease rotational speed. Increase the speed again manually. If necessary, further reduce the target speed until the flow rate visibly runs volume meter.

- Tie a cloth around the hose (Fig. 3, Pos. 1) to prevent it from damage.

- Pinch off this hose 5x consecutively for one second with crimping pliers and re-open. The created interruption and subsequent increase of the flow cause significant movement of the control piston in the flow meter and bleed the measuring system.

- Visually inspect hose (Fig. 3, Pos. 2) of volume meter. Flow rate flows through the volume meter.

- The volume meter is now bled.

Fig. 3: Volume meter B1

Performance check

Check if the measured values are plausible and stable in the course of the performance check.